_____________________________________________________________

04 DEPT - Vehicle Dynamics

SRP-038 – Brakes

Development Process

Sunswift Vision Statement:

Sunswift Racing 8 is a high-performance prototype vehicle that aims to showcase as many cutting-edge technologies as possible. It is versatile in providing a refined driving experience both on the road and around a racetrack. Overall, the car has a high excitement factor that entices people to drive it, defeating the misconception that solar electric vehicles cannot be interesting and engaging.

- Table of Contents

1 - Conceptual Design	4

1.1 - Projects	5

Project Scope	5

Terms, Definitions, and Abbreviations	5

Responsible and supporting departments	5

Project interfaces	5

Assigned team members	6

1.2 - Design Factors	6

Lifecycle factors	7

Risk Analysis	7

Innovation Analysis	7

Manufacture and Procurement	8

Design and Production plan	8

Cost Analysis:	8

Educational Outcome:	8

Upskilling Development:	9

1.3 - Conceptual Requirements	9

Functional requirements	10

Performance requirements	10

Interface requirements	12

External requirements	12

2 - Preliminary Design	13

2.1 - Key Components	14

Key Components definition	14

2.2 - Design Factors	14

Lifecycle	14

Risk	14

Innovation	15

Manufacture and Procurement	15

Design and Production Plan	16

Cost Analysis	16

17

Educational Outcome	17

Upskilling Development	17

2.3 - Preliminary Requirements	17

Functional Requirements	18

18

Performance Requirements	18

Reliability Requirements	18

Interface Requirements	18

External Requirements	18

CAMBER:	20

TOE:	20

References	21

- Document Revisions

| Document Version | Date | Description | Approved by |
| --- | --- | --- | --- |
| 0.1 | 30/10/2022 | Conceptual Design | Declan Watson |
| 0.2 |  | Preliminary Design |  |

- List of Tables and Figures

List of Figures

| Figure number | Title | Page number |
| --- | --- | --- |

List of Tables

Table 1: Terms, Definitions, and Abbreviations	4

Table 2: Risk Analysis Table	5

Table 3: Manufacture and Procurement	6

Table 4: Functional requirements table (Normal conditions)	8

Table 5: Performance requirements (Teamwide specifications)	9

Table 6: Interface requirements (Project/System interfaces)	9

Table 7: External requirements	9

# 1 - Conceptual Design

## 1.1 - Projects

### Project Scope

SRP – 038 is responsible for the development of the Brakes subsystem acting within Vehicle Dynamics. This will consist of from (not including) the pedal through the hydraulic lines, master cylinder and brake booster, through to the callipers and brake discs. Also, Brakes will be responsible for the design of the park brake. The aim of the brakes department is to research viable braking systems and options in order to determine packaging requirements, fundamental for other vehicle systems and importantly the design of suspension uprights.

### Terms, Definitions, and Abbreviations

Table 1: Terms, Definitions, and Abbreviations

| Abbreviation/Terms | Definition |
| --- | --- |
| TPM | Technical performance measures |
| SR8 | Sunswift 8 |
| OTS | Off-the-Shelf |

### Responsible and supporting departments

Vehicle Dynamics (Responsible)

Interior (Supporting)

### Project interfaces

SRP – 038 will interface with the following departments:

Table 1: Terms, Definitions, and Abbreviations

| Relevant Departments | Description |
| --- | --- |
| SRP – 010 Test Mule | Test mule has accurate dimensions (pickup points and barking system dimensions) for effective testing and optimization. |
| SRP – 022 Human Machine Interface | Braking controls that can be adjusted via interfaces on the steering wheel such as brake bias. |
| SRP – 026 Pedals | Providing interior with specific details such as type of pedal as well as percentages of pedal travel |
| SRP – 035 Performance optimisation | Braking performance can be adjusted to suit conditions, optimize braking distance, and minimize braking temperatures. |
| SRP – 040 Steering | Ensuring that braking components (eg lines) do not interfere with steering components. |
| SRP – 041 Suspension | Pick up points and locations that will be necessary for the construction of uprights. |
| SRP – 042 Wheels | Ensure that brake disks and calipers fit within the inside barrel of the wheel. |
| SRP8 - 015 Cruise Control | If it is adaptive, brakes must be used when distance shortens between the car and the object in front of it |
| SRP8 - 098 Automated Parking | Brakes must be used |
| SRP8-059 Sensors | Sensors will be used to monitor pressure and collect data |
| SRP8-014 Regen Braking | Regen braking system must be implemented into the system |

### Assigned team members

- Martin Nguyen (Project Lead, 24T1 – Present)

- Declan Watson

## 1.2 - Design Factors

### Lifecycle factors

The brake system in theory should have a lifespan which mimics that of the vehicle. Disposable components of the braking system include hydraulic lines, piston seals, brake pads and brake discs.  ​

It should be considered throughout preliminary design as the braking system should last over the average lifespan of the typical passenger vehicle.  ​

Typically for passenger vehicles this lifespan is 90,000 km’s for brake pads and 150,000 km’s for brake discs.

### Risk Analysis

Major concerns would surround the braking force and impact of heat. Brakes when heated (more specifically overheated) lack braking performance.  ​

Typical braking performance should be compared among varied materials to optimize braking performance and vehicle safety. ​

Table 2: Risk Analysis Table

| Risk | Response Category | Mitigation Strategy |
| --- | --- | --- |
| Braking Hardware |  |  |
| Brake Fade | Mitigate, Avoid | Focus on a solution which dissipates heat. Reduce the number of heavy braking intervals while on track. |
| Brake Pad (and Disc) wear | Accept | Regular servicing/inspections which is included as a vehicle inspection check. See lifecycle for lifespan of these components. |
| Brake squeaks | Mitigate | Every time the brakes have come into contact with someone, they are to be cleaned with Brake Cleaner. |
| Braking System |  |  |
| Brake leak | Mitigation | Use of braided lines as well as regular inspections of the braking hardware to mitigate splits in brake lines. The use of Teflon tape at all fittings. |
| Brake Booster fails | Servicing, Mitigation | If the Brake Booster is to fail the system will be bypassed and intermechanical pressure will remain between driver and braking hardware |
| Regen |  |  |

### Innovation Analysis

### Material analysis, lifecycle analysis and comparisons will need to be conducted to determine feasible options but also options which are worthwhile.  ​

### This would include the possibility of manufacturing in house of components to justify some decisions.  ​

### It is important to keep in mind that by trying to find innovative solutions it would be best for us to manufacture them rather than buying them off-the-shelf. By doing this factor of safety will be impacted which must be tested to ensure the upkeep of both legal standards but also road-user safety.

### Manufacture and Procurement

This would include the exploration of production of braking components such as brake discs. This would be with the help of McConaughy. Otherwise, off-the-shelf options must be explored.  ​

With the braking system being such a fundamental factor of safety of a passenger vehicle it would require a high level of attention and strict procedures if it were to be manufactured.

Table 3: Manufacture and Procurement

| Item | Method |
| --- | --- |
| Mounting Bell | In house CAM or send to Sorensen or can be purchased OTS. |
| Discs | Purchased OTS |

### Design and Production plan

Key milestones include:

- Preliminary Design

- Detailed Design

- Construction

### Cost Analysis:

The main components that make up the bulk of the cost for the brake system include the callipers, brake discs, master cylinders, IBSe9’s and electronically activated callipers due to their complex design. The cost of the parts can be reduced using partnerships with AP Racing, Comp Friction and LSP. The table below provides an estimation of the costs of each brake system component based on information provided by distributers online.

| Component | Component Code | Quantity | Link | Cost per unit in AUD (Based on approximate prices available) | Total |
| --- | --- | --- | --- | --- | --- |
| Brake callipers | AP Racing CP9200-2S0BG | 2 | https://www.demon-tweeks.com/au/ap-racing-cp9200-4-piston-two-piece-forged-radial-mount-brake-caliper-7000565/ | $963.07 | $1926.14 |
| Brake callipers | AP Racing CP9200-3S0BG | 2 | https://www.demon-tweeks.com/au/ap-racing-cp9200-4-piston-two-piece-forged-radial-mount-brake-caliper-7000565/ | $963.07 | $1926.14 |
| Brake discs | AP Racing CP3047-252/253 | 4 | https://www.demon-tweeks.com/au/ap-racing-ventilated-brake-discs-300-355mm-240005/ | $754.46 | $3017.84 |
| Brake pads | AP Racing CP3215D50 | 4 | https://intimabrakes.com.au/product/intima-sr-md8472-sr/ | $199 | $796 |
| Master cylinders | CP7854 | 2 | https://www.murraymotorsport.com/ap-racing-cp7854-master-cylinder-trunnion-mount-aprcp7854-c | $615.33 | $1230.66 |
| Reservoir | CP2293-141/-143 | 2 | https://www.rpxpress.com/products/reservoir-single-90-outlet-ap-racing-cp2293-143 | $97.43 | $194.86 |
| Brake Lines | - | - | https://aeroflowperformance.com/brake/hose-fittings/hardline | $7.51/m | $57.05 |
| Brake Hose | - | 6 | https://www.vsport.com.au/?rf=kw&kw=brake+lines | $240 | $1440 |
| Brake line pressure sensors | - | 2 | https://microtechefi.com/product/pressure-sensor-0-1500/ | $125 | $250 |
| Residual pressure valve | - | 2 | https://www.proflow.com.au/brake-residual-valve-2lbs-blue-1-8-npt | $21.22 | $42.44 |
| Electronic Park Brake (right) | Wilwood 120-15702 | 1 | https://wilwood.com/calipers/CaliperProd?itemno=120-15702-BK | $563.21 | $563.21 |
| Electronic Park Brake (left) | Wilwood 120-15703 | 1 | https://wilwood.com/calipers/CaliperProd?itemno=120-15702-BK | $563.21 | $563.21 |
| Balance Bar | CP5520-25 | 1 | https://apracing.com/race-car/pedal-boxes/balance-bars/trunnion-types/conventional-mount-high-efficiency-type-cp5520-25 | Already owned | - |
| Brake Booster | IBSe9 | 2 | https://www.lsp-ias.com/our-products/ibse-motorsport-systems | Already owned | - |
| Total |  |  |  |  | $12007.55 |

### Educational Outcome:

If manufacture were to be viable and safety is a factor that can be upheld the process of manufacture would provide an exceptional learning opportunity for Vehicle Dynamics and more broadly the Mechanical team.  ​

This learning experience would require communications with McConaughy and potentially Sorensen to ensure that safety items are met.

The students will be able to:

- Analyze braking performance and assemble braking systems

- Research and source off-the-shelf braking components

- Collaborate efficiently with other departments such as Chassis & Bodywork, Interior

### Upskilling Development:

Upskilling development is all dependent on whether manufacture is viable. If so inductions in Mill and Lathe would be necessary.

- Sheet Metal – Induction needed before mill and lathe inductions

- OptimumG Series

  - OptimumTire

  - OptimumK

## 1.3 - Conceptual Requirements

### Functional requirements

Table 4: Functional requirements table (Normal conditions)

| Reference | Title | Description | TPM |
| --- | --- | --- | --- |
| REQ1.01 | Vehicle Stops | Braking capacity meets standard from calculations. | TPM101 – The braking system stops the vehicle from moving. |
| REQ1.02 | Vehicle Packaging | Brake disc and caliper fits within the barrel of the wheel | TPM102 – Braking package fits within the wheels. |
| REQ1.03 | Vehicle Packaging | Brake lines and master cylinder do not impede other interfaces within the vehicle. | TPM103 – Brake lines and master cylinder have specified locations. |
| REQ1.04 | Vibrations | Withstands all road vibrations. | TPM104 – Brakes handle all road vibrations caused by tarred road travel (Track and public road) |
| REQ1.05 | Braking | Limited brake fade (consistency) | TPM105 – Brake fade is minimal and does not require interruptions during track runs. |
| REQ1.06 | Braking | The brakes should not differ (at least to the feel of the driver) in the wet/dry or cold/hot. | TPM106 – Braking performance is equivalent and consistent despite conditions. |
| REQ1.07 | Service | Braking package can be serviced easily without having to remove multiple parts of the vehicle. | TPM107 – Braking package has easy access for service. |
| REQ1.08 | Enironmental | The system is clean and doesn’t leave behind copious amounts of brake dust. | TPM108 – System has an improvement on ecological cleanliness compared to current passenger vehicles |
| REQ1.09 | Packaging | The entire braking package is lightweight and compact | TPM109 – The entire braking package is compact and lightweight. |
| REQ1.10 | Fail Safe | Two separately plumbed systems can stop the car | TPM110 – There are two systems to stop the car in case of failure. |

### Performance requirements

Table 4: Performance requirements (Teamwide specifications)

Table 5: Performance requirements (Teamwide specifications)

| Reference | Title | Description | TPM |
| --- | --- | --- | --- |
| REQ2.01 | Unsprung mass | Brakes weight should be minimal in order to reduce unsprung mass and enable faster reaction times for SR8 suspension. | TPM201 – Brakes weight should be optimized to be as small as possible. |
| REQ2.02 | Rolling resistance | Rolling resistance of the vehicle should be nominal. Allowing greater efficiency and performance. Brake disc weight and pad retention are considerations for this. | TPM202 – Rolling resistance should be optimized to be nominal. |
| REQ2.03 | Safety | The braking system is responsible for the safety of the occupants and must safely stop the vehicle | TPM203 – Brakes safely bring the vehicle to a stop. |
| REQ2.04 | Braking Capacity | Braking capacity should be made appropriate to meet all functional requirements and as many performance requirements as possible. | TPM204 – Braking capacity should be made to meet as many performance requirements as possible while still meeting all compulsory requirements. |
| REQ2.05 | Track performance | Brakes should optimize performance around the track. In accordance with the SR8 vision. | TPM205 – Brakes are optimized for track performance in the pursuit of the best lap times. |
| REQ2.06 | Road efficiency | Brakes are designed for road driving and optimized for this use case. | TPM206 – Brakes are optimized for road driving in terms of safety, efficiency and durability. |
| REQ2.07 | Innovation | The braking system must be innovative and create excitement around SR8. | TPM207 – Braking system has innovative ideation and distinguishes us from other solar car projects. |
| REQ2.08 | Sustainability | Brakes must be as sustainable as possible in terms of material use and manufacturing. End of life cycles are considered for brake discs and pads. | TPM208 – Brakes are designed to be sustainable. |

### Interface requirements

Table 5: Interface requirements (Project/System interfaces)

Table 6: Interface requirements (Project/System interfaces)

| Reference | Title | Description | TPM |
| --- | --- | --- | --- |
| REQ3.01 | Packaging | Interface between projects. Close collaboration with Packaging to enable other departments to interface with brakes and to determine the space in which brakes fit and operate. | TPM301 - Packaging requirements for brakes are met to ensure no impedance to braking performance and drivability of other departments. |
| REQ3.02 | Test Mule | Test mule has accurate dimensions (pickup points and braking system dimensions) for effective testing and optimization. | TPM302 - Test mule has accurate dimensions (pickup points and braking system dimensions) for effective testing and optimization. |
| REQ3.03 | Human Machine Interface | Braking controls that can be adjusted via interfaces on the steering wheel such as brake bias. | TPM303 – Braking controls that can be adjusted via interfaces on the steering wheel such as brake bias. |
| REQ3.04 | Pedals | Providing interior with specific details such as type of pedal as well as percentages of pedal travel. | TPM304 - Providing interior with specific details such as type of pedal as well as percentages of pedal travel. |
| REQ3.05 | Steering | Ensuring that braking components (e.g. lines) do not interfere with steering components. | TPM305 - Ensuring that braking components do not interfere with steering components. |
| REQ3.06 | Suspension | Pickup points and locations that will be necessary for the construction of the uprights. | TPM306 – Discussions about pickup points and locations that will be necessary for the construction of the uprights. |
| REQ3.07 | Wheels | Ensuring that the brake discs and calipers fit within the inside barrel of the wheel. | TPM307 - Ensuring that the brake discs and calipers fit within the inside barrel of the wheel. |

### External requirements

Table 6: External requirements

Table 7: External requirements

| Reference | Title | Description | TPM |
| --- | --- | --- | --- |
| REQ4.01 | Regen and Mechanical Braking (ADR 2.15 & 2.17) | Brakes function both under regen and mechanical braking. | TPM401 – Brakes function both under regen and mechanical braking. |
| REQ4.02 | Vibrations (ADR 5.1.1.1.) | Brakes continue to function without change under vibrations. | TPM402 – Brakes continue to function without change under vibrations. |
| REQ4.03 | Fail Safe (ADR 5.1.2.2.) | There is a secondary braking system in play in the vehicle. | TPM403 – There is a secondary braking system in play in the vehicle. |
| REQ4.04 | Park Brake (ADR 5.1.2.3.) | The park braking system functions at specific angles of inclination and declination. | TPM404 - Park braking system functions at specific angles of inclination and declination. |

Notes: vehicle category code defined by ADR: “MA” for passenger car. ADR01/00 – 01 designates number, 00 designates revision.ADR’s (M1): ADR31.pdf

# 2 - Preliminary Design

## 2.1 - Key Components

### Key Components definition

Facilitate the movement of SR8 while supporting the load presented by all subsystems. Primarily applying the torque or braking forces applied to SR8 onto its environment as required. Maintaining energy efficiency, stability and limiting rolling resistance.

| Project: | Front Brakes |
| --- | --- |
| Key Components: | Brake calipers (inc. Dog bone) Brake disc (inc. Mounting bell) Brake pads Master cylinder Reservoir IBSe Hard brake lines Braided brake lines Brake fluid Fittings (banjos, elbows, t-pieces) |

NOTE: Balance Bar is included yet it is what interfaces between front brakes and rear brakes.

| Project: | Rear Brakes |
| --- | --- |
| Key Components: | Brake calipers (inc. Dog bone) Brake disc (inc. Mounting bell) Brake pads Master cylinder Reservoir IBSe Hard brake lines Braided brake lines Brake fluid Fittings (banjos, elbows, t-pieces) |

| Project: | Park Brake |
| --- | --- |
| Key Components: | Brake Caliper (separate caliper potentially which is then added to the rears) Actuation (electric motor) |

## 2.2 - Design Factors

### Lifecycle

Components are considered to be in one of three categories:

- Off-the-shelf components: Will have a lifecycle revolving around product selection.

- Product Selection

- Fitment & Assembly

- Testing & Maintenance

- Afterlife

- House-made components: Will have a lifecycle revolving around design and production.

- Design & Analysis

- Machining

- Fitment & Assembly

- Testing & Maintenance

- Afterlife

- Repurposed components: These are components already used and/or purchased which will be used for SR8

- Collation of Data

- Review of previous life

- Design Integration

- Fitment & Assembly

- Testing & Maintenance

- Afterlife

| Key component | Lifecycle |
| --- | --- |
| Brake Calipers | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |
| Dog Bone | Cat. 2: These will be made to preserve the calipers and the uprights. This will be made from an alloy and anodized for increased life. |
| Brake disc | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |
| Mounting Bell | Cat. 2: This will be made to promote the life of the discs and calipers which are often impacted by heat. This will be made from an alloy and anodized for increased life. |
| Brake pads | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |
| Master cylinder | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |
| Balance bar | Cat. 3: An unused part which was additionally purchased. This part is from the AP Racing range and will interface with all AP Racing components. |
| IBSe | Cat. 3: Previously used for the Remote Driving project the IBSe’s will be the brake-by-wire interface responsible for also boosting the brake pressure. |
| Reservoir | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |
| Hard brake line | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |
| Braided brake line | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |
| Brake fluid | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |
| Fittings | Cat. 1: These will be purchased from the AP Racing catalog range and follow the recommended service life maintenance schedule. |

### Risk

### Innovation

The brakes project has experienced low to medium innovation. A number of the key components have been analysed, compared to different options and justified. A few key components will be manufactured impacting the factor of safety which will need to be tested to ensure legal standards and road-user safety is kept.

- Brake discs – medium innovation

  - Bolted disc was chosen off the shelf

  - Manufacture of the mounting bell – medium innovation

  - Reason: Allows for student involvement

- Handbrake – high innovation

  - Hand brake chosen overws2a electronic brake

  - Manufacturing in house

  - Reason: Allows for student involvement

- Hard and braided brake lines chosen – low innovation

  - Will be sourced off the shelf

  - Reason: manufacturing not possible by students

- Pull type master cylinder – low innovation

  - Either CP6461 or CP6465

  - Sourced off the shelf from AP Racing

  - Reason: technical limitations (high failure risk/safety issues)

- Front and rear callipers – low innovation

  - Chosen off the shelf from AP Racing

  - https://apracing.com/race-car/brake-calipers/pro-5000-r-range/4-piston-cp9440-2-3s4l-front-4712cm2-piston-area?switch=0

  - https://apracing.com/race-car/brake-calipers/pro-5000-r-range/4-piston-cp9441-2-3s4l-rear-3619cm2-piston-area?switch=0

  - Reason: manufacturing not possible by students

- Brake pads – low innovation

  - Sourced off the shelf from AP Racing

  - https://apracing.com/race-car/brake-pads/4-piston-caliper-pad-profiles/cp3215d50

  - https://apracing.com/race-car/brake-pads/4-piston-caliper-pad-profiles/cp3215d50

  - Reason: manufacturing not possible by students

### Manufacture and Procurement

### Design and Production Plan

### Cost Analysis

### Educational Outcome

### Upskilling Development

## 2.3 - Preliminary Requirements

### Functional Requirements

Emergency Scenarios

#### User Error

#### Testing/Maintenance/Fault diagnostic

### Performance Requirements

#### Teamwide specifications

#### Technical specifications

#### Safety specifications

### Reliability Requirements

### Interface Requirements

#### Project/System interfaces

#### Mounting points/Area/Routing

### External Requirements

Design comparison

# References

Quiroga, T. 2010, Effects of Upsized Wheels and Tires Tested, Car and Driver, <https://www.caranddriver.com/features/a15130598/upsized-wheels-tires/>